Chemical Hygiene Plan

INTRODUCTION	2

I  ROLES AND RESPONSIBILITIES	3

A  Principal Investigator (PI)	3

B  Laboratory Employees	3

C  Chemical Hygiene Officer (CHO)	3

II  EMPLOYEE INFORMATION AND TRAINING	4

III  PRIOR APPROVAL CIRCUMSTANCES	4

IV  STANDARD OPERATING PROCEDURES	5

A  General Rules	5

B  Personal Hygiene	5

C  Housekeeping	5

D  Protective Clothing and Equipment	6

E  Flammable Materials	8

F  Reactive Chemicals	8

G  Corrosive Chemicals	8

H  Acids and Bases	10

I  Compressed Gas Cylinders	11

V  CONTROL MEASURES	12

A  Ventilation	12

B  Spill Clean-up Procedures	13

VI  EXPOSURE MONITORING	14

VII  MEDICAL CONSULTATIONS AND EXAMINATIONS	14

VIII  SELECT CARCINOGENS, REPRODUCTIVE TOXINS, HIGHLY ACUTE TOXINS	14

Appendix 1  Laboratory Specific Operating Procedures	16

Appendix 2  Laboratory Inspection Checklist	22

Appendix 3  Laboratory Safety Training Record	27

Appendix 4  Lab Close Out Survey Checklist	28

Appendix 5  In Case of Emergency	29

Appendix 6  Hazardous Materials Pickup and Disposal Procedures	30

Appendix 7  Select Carcinogens	33

Appendix 8  Laboratory Emergency Procedures During Power Outages	37

Appendix 9  Requirements for Storage and Handling of Flammable and Combustible Liquids	38

Appendix 10  Chemical Inventory	40

NOTE:  You can save this document in your preferred Word Processer by clicking on the Edit menu, choosing "select all" then click on Copy.  Paste it into your word processing program and edit for your laboratory specific Standard Operating Procedures.

The text in brackets {} are formatted for the table of contents.  The text will not show up when printed.

INTRODUCTION

The University of Alaska Fairbanks (UAF) encourages and supports all programs which promote safety, good health, and well being of UAF faculty, staff, students, participants in UAF sponsored programs, and visitors.  It is the policy of UAF to provide safe and healthful conditions and to reduce injuries and illnesses to the lowest possible level.  No task is so important and no service so urgent that it cannot be done safely.  In keeping with this commitment, this Chemical Hygiene Plan was developed as part of the Laboratory Safety Program.

The Chemical Hygiene Plan (CHP) is designed to protect laboratory personnel from potential hazards associated with the use of chemicals.  Compliance is mandatory for all employees working in campus laboratories due to requirements of the Occupational Safety and Health Administration (OSHA) standard on "Hazardous Chemicals In Laboratories".  While these regulations pertain specifically to employees, provisions of the CHP may apply to students and visitors depending on their activities.

A variety of hazardous chemicals are used in small quantities in research and teaching laboratories creating a unique environment with a number of risks.  These chemicals may cause injury or damage because they are toxic, flammable, corrosive, or reactive with water and other materials.  How these substances are handled will determine the degree of risk.

The objective of this CHP is to provide uniform requirements for safe use of potentially hazardous substances in UAF laboratories.  General standard operating procedures are outlined, including work with select carcinogens, reproductive toxins, and highly acute substances.

Specific standard operating procedures developed by each lab for operations posing a special hazard (for example, heating perchloric acid, working with pyrophorics, conducting electrophoresis, distillations, extractions, etc.) must be developed for your laboratory specific SOP's (Appendix 1).

Maintaining a safe and healthy environment in the laboratory is ultimately the responsibility of the Chemical Hygiene Officer (CHO).  However, each individual is expected to conduct all operations and procedures involving chemicals in a safe and prudent manner.

Basic Instructions

- Assign PI and Lab Supervisor (page 3 of the CHP)

- Complete Appendix 1 for each lab’s Standard Operating Procedures.  Then, return a copy of Appendix 1 to Environmental Health & Safety (EH&S)  (fnljb1@uaf.edu)

- Complete the Chemical Inventory Worksheet (excel spreadsheet).  Then, return a copy of the inventory to EH&S (fnljb1@uaf.edu).

- If any chemicals are present in the lab that are highlighted in yellow on the inventory worksheet, a Laboratory Hazard Statement form must be completed and included in Appendix 1.  A blank form is available in Appendix 1 of this CHP.

- Retain a copy of the CHP and inventory in the lab it pertains to.

- ROLES AND RESPONSIBILITIES

  - Principal Investigator (PI)

(PI:____________________ Lab Supervisor: ________________)

The PI has responsibility for implementation of the Chemical Hygiene Plan (CHP) in his/her laboratory. The PI shall:

    - ensure that workers are trained and follow the CHP outlined in this document;

    - ensure that the necessary protective and emergency equipment is available, in working order, and that appropriate training has been provided;

    - ensure that periodic laboratory inspections are performed (refer to Appendix 2);

    - know current legal requirements concerning regulated substances;

    - review and evaluate the effectiveness of the laboratory specific Standard Operating Procedures (SOP) at least annually and update as necessary.

  - Laboratory Employees

Laboratory employees are responsible for:

    - planning and conducting each operation in accordance with practices and procedures established in this CHP;

    - using equipment only for its designed purpose;

    - being familiar with emergency procedures, including knowledge of the location and use of emergency equipment for the laboratory, as well as how to obtain additional help in an emergency;

    - knowing the types of protective equipment available and using the proper type for each procedure;

    - being alert to unsafe conditions and actions and calling attention to them so corrections can be made as soon as possible.

  - Chemical Hygiene Officer (CHO) is an employee designated by the employer who is qualified by training or experience to provide technical guidance in the development and implementation of the provisions of the CHP.  The current Chemical Hygiene Officer for the building/lab is designated by the PI.

The Chemical Hygiene Officer shall:

    - assist PI's and other laboratory employees with development and implementation of appropriate chemical hygiene procedures and practices, including providing consultation and information;

    - keep abreast of legal requirements concerning regulated substances and communicate any changes to PI's and laboratory employees.

    - seek ways to improve the overall chemical hygiene program.

- EMPLOYEE INFORMATION AND TRAINING

PI's shall ensure that information and training are provided at the time of an employee's initial assignment to a work area where hazardous chemicals are present and prior to assignments involving new exposure situations.  Refresher information and training shall be conducted at least annually and documented.

  - Information

All laboratory personnel shall be informed of:

    - Requirements of the OSHA Standard, "Occupational Exposure to Hazardous Chemicals in Laboratories", 29 CR 1910.1450

    - The contents and availability of this Chemical Hygiene Plan,

    - Permissible Exposure Limits (PELs) for OSHA regulated substances or recommended exposure limits where there is no applicable OSHA standard,

    - Signs and symptoms associated with exposures to hazardous chemicals used in their laboratory,

    - The location of reference materials on the hazards, safe handling, storage and disposal of hazardous chemicals found in the laboratory including, but not limited to, Material Safety Data Sheets (MSDS's).

  - Training

Employee training shall include:

    - The physical and health hazards associated with chemicals stored and used in their work area,

    - The contents of this Chemical Hygiene Plan,

    - Methods and observations that may be used to detect the presence or release of a hazardous chemical; e.g., exposure monitoring conducted by the CHO, visual appearance or odor of hazardous chemicals when being released, etc.

    - The measures employees can take to protect themselves from these hazards, including specific procedures the employer has implemented to protect employees from exposure to hazardous chemicals, such as appropriate work practices, emergency procedures, and personal protective equipment to be used.

- PRIOR APPROVAL CIRCUMSTANCES

Employees must obtain prior approval to proceed with a laboratory task from the PI or his/her designee when:

  - Radioactive materials will be used,

  - Recombinant DNA or biological material of Biosafety Level 3 or greater will be used,

  - It is likely that exposure limit concentrations could be exceeded or that other harm is likely,

  - There is failure of any equipment used in the process, especially of safeguards such as chemical fume hoods.

  - Members of the laboratory staff become ill, suspect that they or others have been exposed, or otherwise suspect a failure of any safeguards.

- STANDARD OPERATING PROCEDURES

  - General Rules

    - For chemicals they are working with, all employees should know:

      - the chemical's hazards, as determined from a MSDS and other appropriate references;

      - appropriate safeguards for using that chemical, including personal protective equipment;

      - how to properly store the chemical when it is not in use;

      - proper chemical waste disposal procedures (Appendix 6);

      - proper personal hygiene practices;

      - appropriate procedures for emergencies, including first aid, evacuation routes, and spill cleanup procedures.

    - Employees should avoid working alone.  Arrangements should be made between individuals working in separate laboratories outside of regular working hours to crosscheck each other periodically.  Experiments known to be hazardous should not be undertaken by an employee who is alone in the laboratory.

  - Personal Hygiene

    - Wash promptly whenever a chemical has contacted your skin.  Flush for at least 15 minutes prior to seeking medical attention.

    - Avoid inhalation of chemicals.  Do not "sniff" to test chemicals.

    - Do not use mouth suction to pipette anything.  Pipetting aids must be used at all times.

    - Do not bring food (including gum and candy), beverages, tobacco, or cosmetic products into chemical storage or use areas.  Eating, drinking, and applying cosmetics is allowed in designated areas only.  Smoking is prohibited in all UAF facilities.

    - Wash well with soap and water before leaving the laboratory.  Avoid the use of solvents for washing skin.  Solvents remove the natural protective oils from skin and can cause irritation and inflammation.  In some cases, washing with solvent may facilitate absorption of toxic chemicals.

  - Housekeeping

Housekeeping is directly related to safety and must be given importance of equal value to other procedures.  Lack of good housekeeping reduces work efficiency and may result in accidents.  Laboratory personnel must adhere to the following:

    - Access to emergency equipment, showers, eyewashes, fire extinguishers, exits and circuit breakers shall never be blocked or obstructed.

    - Chemical containers should be regularly monitored for proper labeling and container integrity.  Labels which are fading, falling off, or deteriorating must be promptly replaced.  If abbreviations are used, they should be kept to a minimum and clearly identify the contents of the container as well as hazards associated with use; i.e., HgCl2/poison, HCl/corrosive, MeOH/flammable, H2O2/corrosive-oxidizer, Nonhazardous buffer, etc.  Improperly labeled or unlabeled chemicals make hazard identification and disposal difficult, and may create a hazard.

    - All chemicals should be placed in their proper storage areas at the end of each workday.  Chemicals shall not be stored on desks, laboratory benchtops, floors, fume hoods or in aisles.

    - Each laboratory must have a puncture resistant (e.g., cardboard) container specifically designated for glassware disposal.

    - At the end of each workday, the contents of all unlabeled containers are to be considered waste and disposed of appropriately.

    - Collection containers for wastes must be clearly labeled including hazard identification.

    - All work areas, especially laboratory benchtops, should be kept clear of clutter.

    - All aisles, corridors, stairs, and stairwells shall be kept clear of chemicals, equipment, supplies, boxes, and debris.

    - Food and drink for human consumption shall not be kept in the same refrigerator used to store chemicals and laboratory samples.  Eating and office areas must be clearly separated from laboratory and chemical storage areas.

    - Empty containers shall be treated in the following manner:

      - For water soluble solvents: triple rinse, deface the label, relabel as "Empty" and dispose with normal trash.

      - For non-water soluble solvents: triple rinse using a solvent capable of removing the chemical. ALL rinsate must be collected in a hazardous waste disposal container.  Deface the label, relabel as "Empty" and dispose with normal trash.

  - Protective Clothing and Equipment

    - Carefully inspect all protective equipment prior to use.  Do not use defective equipment.

    - Eye protection (safety glasses, chemical-resistant goggles, or face shield) shall be worn at all times in laboratories where chemicals are being used.  This includes visitors.  Ordinary prescription glasses are not considered effective eye protection since they lack necessary shielding.  Chemical-resistant goggles should be worn over the glasses or prescription safety glasses be provided to employees required to wear corrective lenses.

    - The wearing of contact lenses in the laboratory is very controversial.  Consult with an Optometrist prior to wear in the laboratory.  Safety glasses or chemical-resistant goggles shall be worn over contacts at all times.

    - When working with corrosive, toxic, allergenic, or sensitizing chemicals, rough or sharp-edged objects, very hot or very cold materials, gloves made of material known to be resistant to permeation by the substance shall be worn.  No one glove can protect against all hazards.  Cloth gloves, while not appropriate for use around liquids, can protect against light abrasive materials and moderate temperature changes.  Synthetic or rubber gloves protect against corrosives, solvents, and poisons.  Leather gloves, often used for tasks like welding, protect against sparks, heat, and rough abrasives.

Consult the manufacturer's performance chart or contact the Chemical Hygiene Officer to determine the proper choice of glove material.

    - Low-heeled shoes with fully covered uppers shall be worn at all times in the laboratory.  Shoes or sandals with open toes shall not be worn.

    - Long pants and long sleeves should be worn when working with or around chemicals.

    - Long hair should be held in place behind the head.

    - Loose clothing, especially loose trouser legs and sleeves, should not be worn in the laboratory.

    - A full-body-length rubber, plastic, or neoprene apron appropriate for the material being handled should be worn if there is risk of splash or spill.

    - A proper respirator must be worn whenever exposure by inhalation is likely to exceed the action level or Personnel Exposure Limit (PEL) and a fume hood is not accessible.  Procedures specified in the UAF Respiratory Protection Program must be followed.  Employees must be medically qualified, trained, and fit-tested prior to using a respirator.  Consult your PI and/or the CHO before doing any such work.

  - Flammable Materials

Precautions for safe handling of flammable materials include the following:

    - Storage of flammable substances shall be limited to quantities specified in Appendix 9.

    - Flammable substances shall be handled only in areas free of ignition sources.

    - Flammable substances should never be heated by using an open flame. Preferred heat sources include steam baths, water baths, oil baths, heating mantles, and hot air baths.

    - Class I liquids (Appendix 9) shall not be transferred from one vessel to another in any exit way.

    - Transfer of flammable liquids from 5 gallon containers (or less) to smaller containers shall be conducted in a laboratory fume hood or an approved flammable liquid storage room.

  - Reactive Chemicals

A reactive chemical is one that:

    - Fits the OSHA definition of "unstable" in 1910.1450(b):

"Unstable (reactive) means a chemical which in the pure state, or as produced or transported, will vigorously polymerize, decompose, condense, or will become self-reactive under conditions of shocks, pressure, or temperature

    - Is ranked by the National Fire Protection Association (NFPA) as 3 or 4 for reactivity,

    - Is identified by the Department of Transportation (DOT) as:

      - An oxidizer,

      - An organic peroxide, or

      - A class A, B, or C explosive,

    - Violently reacts with exposure to water or air.

Handle reactive chemicals with all proper safety precautions. This includes designating a separate storage area, monitoring periodically for degradation, and using appropriate personal protection.

  - Corrosive Chemicals

    - Materials are classified as corrosive if they:

      - are capable of rapidly eroding building materials or metals, or

      - burn, irritate or destructively attack organic tissues such as skin, eyes, lungs and stomach.

Examples of commonly used chemicals that have corrosive properties are:

acetic anhydride		bromine			chlorine

fluorine			glacial acetic acid		hydrochloric acid

hydrofluoric acid		nitric acid			potassium hydroxide sodium hydroxide	sulfuric acid

Safe handling procedures will vary with each operation and the type and concentration of the corrosive chemical.

  - Acids and Bases

    - The following general guidelines should be followed for procedures involving acids and bases:

      - Never pour water into acid.  Slowly add the acid to the water and stir.

      - Open bottles or carboys slowly and carefully, wearing protective equipment to guard hands, face, and body.

      - OSHA requires suitable facilities, such as a safety shower and eyewash, to be located within the work area.  The American National Standards Institute (ANSI) recommends that the safety shower and eyewash be within 100 feet of the work area for quick drenching or flushing of the eyes and body.  Eyewash stations and showers should be tested by annually by Facility Services.

      - Procedures requiring the use of concentrated acids and bases must be conducted in a fume hood.

      - Never mix acid wastes with other materials such as solvents, metal-contaminated solutions, etc.  Noncontaminated acid wastes can be easily disposed by neutralization.  Specific procedures are outlined in Appendix 1.

      - When disposable containers are completely emptied of their contents, flush them thoroughly with water before throwing them away.

      - Never dispose of acids or bases in the sanitary sewer system (i.e., down the drain) until neutralized (pH 6.0-8.0). Neutralization may be conducted in the laboratory when included as part of an experiment, and should be conducted in a fume hood. The solution should then be poured slowly down the drain with copious amounts of water; i.e., leave the water running for approximately 5 minutes.

      - Contact Hazardous Materials Division, x5487, for assistance with disposal of large quantities (more than 1 gallon or 1/2 pound) of acids and bases.

      - Each laboratory should have access to a spill kit that includes acid and base neutralizer; follow Spill Response Procedures in section V. B.  Never use combustible organic materials (sawdust, excelsior, wood scraps and shavings, paper, rags, or burlap bags) to absorb or cleanup spillage.

  - Compressed Gas Cylinders

Use of compressed gases in the laboratory requires anticipating chemical, physical, and health hazards.  Cylinders that are knocked over or dropped can be very dangerous.  If a valve is knocked off, the cylinder can become a lethal projectile.  Accidental releases may result in an oxygen depleted atmosphere or adverse health effects.  In short, improper handling and use can cause structural damage, severe injury, and possibly death.

The following guidelines will help ensure safe handling, use, and storage of compressed gas cylinders.

RECEIVING AND STORAGE

    - Be sure to arrange a return agreement with suppliers prior to purchase since disposal of compressed gas cylinders is difficult and very expensive.

    - Cylinders should not be accepted unless the cylinder contents are clearly labeled.  Color code only should not be accepted, since it does not constitute adequate labeling.

    - Do not accept cylinders which are damaged or do not have a valve protection cap.

    - All gas cylinders in use shall be secured in an upright position in racks, holders, or clamping devices.  When cylinders are grouped together, they should be individually secured and conspicuously labeled on the neck area.

    - Oxygen cylinders shall never be placed near highly combustible materials, especially oil and grease, or near stocks of carbide and acetylene or other fuel gas cylinders, nor near any other substance likely to cause or accelerate a fire.  Systems and components used for other gases and purposes must never be used for oxygen or interconnected with oxygen.

    - Cylinders should have current hydrostatic test date (normally less than 5 years old for steel and 3 years old for aluminum) engraved on the cylinder.  Cylinders should be returned to the supplier for servicing prior to the expiration date.

    - Do not place cylinders near heat, sparks, or flames or where they might become part of an electrical circuit.

    - Do not store cylinders in exit corridors or hallways.

HANDLING AND USE

    - Only Compressed Gas Association fittings and components are permitted for use with gas cylinders.  Only use regulators approved for the type of gas in the cylinder.  Do not use adapters to interchange regulators.

    - Open cylinder valves slowly and away from the direction of people (including yourself).  Never force a gas cylinder valve.  If the valve cannot be opened by the wheel or small wrench provided, the cylinder should be returned.

    - No attempt shall be made to transfer gases from one cylinder to another, to refill cylinders, or to mix gases in a cylinder in the laboratory.

    - All cylinders are to be considered full unless properly identified as empty by the user.  Empty cylinders must be returned to the supplier and not accumulated.

    - Compressed gases must not be used to clean your skin or clothing.

    - Never heat cylinders to raise internal pressure.

    - Do not use copper (>65%) connectors or tubing with acetylene.  Acetylene can form explosive compounds with copper, silver, and mercury.

    - Always leave at least 30 psi minimum pressure in all "empty" cylinders.  Do not leave an empty cylinder attached to a pressurized system.

- CONTROL MEASURES

  - Ventilation

    - Laboratory ventilation is normally designed to provide a minimum of eight air changes per hour.  This flow is not necessarily sufficient to prevent accumulation of chemical vapors.  Laboratory work shall be conducted in a fume hood, glove box, or similar device when:

      - Procedures call for work with toxic substances which are volatile; i.e., evaporate at normal temperature and pressure, or

      - There is a possibility the action level or PEL will be exceeded.

    - The protection provided by laboratory fume hoods is dependent upon two important factors:

      - proper use of the hood, and

      - maintenance of adequate airflow through the hood.

    - The way the hood is used will determine the degree of protection it will provide.  Each employee is responsible for implementing the following work practices when using a hood.

      - Continually monitor air being drawn into the hood. This can be done by attaching a light-weight strip of paper to the bottom of the sash.

      - Operate the hood at a sash position that will provide splash protection for the user; e.g. 10-12 inch opening for hoods with vertical sliding (up and down) sashes and the sashes closed as much as possible for continuous air flow hoods with horizontal sliding (left and right) sashes.  This helps to ensure optimum protection when conducting operations in the hood.

      - Avoid using the hood for storage of bottles and equipment, especially along the back wall.  Any apparatus that must be housed within the hood should fit completely inside the hood.  Elevate the apparatus on blocks ( at least 2 inches off the benchtop) to allow air to flow freely around and beneath.

      - Manipulations within the hood should be performed at least 6 inches inside the face of the hood or as far towards the back of the hood as possible.  This minimizes the possibility of contaminants escaping from the hood.

      - Things which cause air turbulence across the face of the hood such as fans, window air conditioning units, or excessive movement should be avoided.

      - Exhaust hoods do not provide adequate protection for all operations involving toxic materials.  A higher level of containment should be used for procedures where minor contamination can be serious.  If you are in doubt about the level of containment needed for your operation, ask your PI, Lab Supervisor, or contact the CHO.

    - Fume hoods should be performed annually to verify adequate airflow is maintained through the hood face.  Departments can do this themselves by borrowing EH&S testing equipment or call EH&S to schedule an appointment.  Face velocities should be between 80 and 125 feet per minute (fpm).    Contact EH&S at x5413 if you suspect the hood is not working properly.

    - In the event of a fume hood failure or low-flow alarm, discontinue all fume hood operations and, only if it is safe to do so, place lids on open containers, lower the hood sash and secure reactions that may be generating hazardous emissions.  Contact Facilities Services Dispatch (extension 7000) to report the alarm after following the steps listed above.  After work hours, this number is forwarded to Emergency Dispatch so essentially, x 7000 is manned 24 hours.  If the danger level is immanent, leave the lab immediately.  If conditions warrant, i.e. concentrations are not higher than the LEL, activate the fire alarm via the pull down stations that are located near building exits. When arriving in a safe area, call 911.

  - Spill Clean-up Procedures

    - Attend to anyone who may have been contaminated.

    - Notify occupants in the immediate area about the spill.

    - Evacuate all nonessential personnel from the spill area.

    - If the spilled material is flammable, turn off all ignition and heat sources; this includes magnetic stirrers.

    - Avoid breathing vapors of the spilled material.  If medically qualified and trained as required in the Respiratory Protection Program, use a respirator if necessary.

    - Ensure that the fume hood(s) is on. Open windows where possible to increase exhaust ventilation.

    - Secure cleanup supplies. Ensure protective apparel is resistant to the spill material.

    - Confine or contain the spill to a small area.

    - Use appropriate kit to neutralize and absorb inorganic acids and bases.

- EXPOSURE MONITORING

Exposure monitoring shall be performed when there is reason to believe that exposures are in excess of the action-level or the PEL.  Materials which require monitoring under these conditions are listed in OSHA Regulations.  If an employee would like to have an exposure assessment conducted, the CHO should be contacted.  Exposure assessments and monitoring may be conducted by the CHO, Facility Services, or UAF Industrial Hygienist.

Documentation of exposure monitoring shall be kept and maintained as part of each employee's personnel record.

- MEDICAL CONSULTATIONS AND EXAMINATIONS

Employees shall be provided an opportunity to receive medical attention, including any related follow-up examinations, at UAF's expense, under the following circumstances:

  - An individual develops signs or symptoms associated with exposure to hazardous chemicals in the laboratory.

  - Exposure monitoring reveals an exposure level routinely above the action level or PEL for an OSHA regulated substance for which there are exposure monitoring and medical surveillance requirements.

  - An accident such as a spill, leak, equipment failure, or explosion results in possible over-exposure to hazardous chemicals.

The PI is responsible for establishing and maintaining an accurate record of any medical consultations and examinations provided to an employee.

- SELECT CARCINOGENS, REPRODUCTIVE TOXINS, HIGHLY ACUTE TOXINS

The procedures described in this section are mandatory when performing laboratory work with greater than 10 mg or 100 mL of any carcinogen, reproductive toxin, or substance that has a high degree of acute toxicity.

  - Definitions

    - Select carcinogens: any substance defined as such by OSHA (refer to Appendix 7).

    - Reproductive toxin: chemicals which affect reproductive capabilities including chromosomal damage (mutations) and effects on fetuses (teratogenesis).

    - Highly Acute Toxin is any substance for which:

      - the median oral LD50 is less than or equal to 50 mg/kg when administered orally to albino rats, or

      - the median inhalation lethal concentration, LC50, value is less than or equal to 200 ppm by volume of gas or vapor, or 2 mg/liter or less of dust, mist, or fume when administered continuously for one hour or less to albino rats, or

      - the median LD50 is less than or equal to 200 mg/kg when administered by continuous contact for 24 hours or less with the bare skin of albino rabbits.

    - Designated area: a hood, glove box, portion of a laboratory, or an entire laboratory room, designated as the only area where work shall be conducted with quantities of select carcinogens, reproductive toxins, or highly acute toxins in excess of the limits specified above.

  - Designated Area

Access to designated areas shall be restricted. Only trained employees will be allowed to work with chemicals in the designated area. All such persons will:

    - Use the smallest amount of chemical that is consistent with the requirement of the work to be done.

    - Always use these chemicals in a hood with adequate air flow (face velocity between 80 and 120 feet per minute) or other containment device for procedures which may result in the generation of aerosols or vapors containing the substance.

    - Use high-efficiency particulate air (HEPA) filters or high-efficiency scrubber systems to protect vacuum lines and pumps.

    - Decontaminate designated areas before normal work is resumed there. This includes contaminated equipment.

    - Remove any protective apparel, place it in an appropriately labeled container, thoroughly wash hands, forearms, face, and neck on leaving a designated area.

    - Prepare wastes for disposal in accordance with Chemical Waste Disposal Guidelines

    - Do not wear jewelry when working in designated areas since decontamination of jewelry may be difficult or impossible.

Appendix 1

Laboratory Specific Standard Operating Procedure

In addition to the Chemical Hygiene Plan Template provided here, you must include your own laboratory specific SOP's in this Appendix

Examples of information to provide include:

- Specialized Laboratory Equipment Description Policies, Procedures and Operating Requirements.  Such equipment includes but is not limited to: Ventilation, lasers, Atomic Absorption, Muffle furnaces, etc.

- Specific standard operating procedures developed by each lab for operations posing a special hazard (for example, heating perchloric acid, working with pyrophorics, conducting electrophoresis, distillations, extractions, etc.

- A reference to the UAF Chemical Inventory Worksheet that should be attached as Appendix 10.  Please call 5812 or 5487 if you do not have this worksheet.

- For each chemical used in the lab that is also highlighted in yellow on the Inventory Worksheet, a Specific Laboratory Hazard Statement is required and should be included in this Appendix.  A blank form is on the next page.

- Specific procedures neutralizing noncontaminated acid wastes.

- Specific clean-up procedures for the different types of chemical in used in the lab.  This should also include where the clean-up materials are stored.

- Training requirements of lab personnel.  Documents should record who will be the trainer, topics to be covered, and the frequency of the training.

- Methods and frequency of conducting chemical inventories including who will perform them, where the inventory is listed is kept, and how the list will be updated.

- Policy of disposing materials when the shelf life has expired.

- Policy for storing chemicals that require special attention such as refrigeration, freezing, acid cabinets, flammable cabinets, etc.

- Specific procedures for operations that are to be conducted in fume hoods.

- Any procedures in place for transporting chemicals.

- Procedures during Emergency Power Outages as referenced in Appendix 8.

- Any other laboratory specific procedure that is required.

Portions of these requirements may be omitted if they are adequately covered in the Departmental Laboratory Policies & Procedures.

Specific Laboratory Hazard Statement

This hazard statement is to be completed for any chemical present in the lab that is highlighted in yellow on the chemical inventory worksheet.

The following information is provided to communicate potentially hazardous operations and the actions taken to mitigate their associated hazards.

Department Name:__________________________________________________

Lab Location:  Building ____________________________  Room ___________

Laboratory Contacts:

Name: __________________ Work Phone__________ Home Phone _________

Name: __________________ Work Phone__________ Home Phone _________

Name: __________________ Work Phone__________ Home Phone _________

Project Description (please include quantity and concentration of chemical): ________________________________________________________________________________________________________________________________________

____________________________________________________________________________________________________________________________________________________________________________________________________________

________________________________________________________________________________________________________________________________________

Anticipated Duration of Project: __________________________________________

Laboratory Process or Equipment: ____________________________________________________________________________________________________________________________________________________________________________________________________________

________________________________________________________________________________________________________________________________________

Approval Required:	Yes ____________	No _____________

Principal Investigator: _______________________________  Date __________

Lab Supervisor: ___________________________________   Date __________

Chemical Information

Chemical Name: _____________________________  CAS:_________________

Quantity _____________		LEL:____________		UEL:______________

Physical State   Solid ____    Liquid ____     Gas ____ 	Semi-solid _____

Freezing Point:________	LD50:_______	Fire Code Classification:___________

Boiling Point:_______	LC50:_______	PEL:________________

Fumehood Use Only:   Yes  _____	No _____

Hazard Type (check all that apply)

| ____ Toxic | ____ Highly Toxic | ____ Carcinogen | ____ Teratogen |
| --- | --- | --- | --- |
| ____ Mutagen | ____ Biohazardous | ____ Radioactive | ____ Explosive |
| ____ Flammable | ____ Oxidizer | ____ Corrosive | ____ Reactive |
| ____ Pyrophoric | ____ Water Reactive | ____ Peroxide Former |  |

Special Precautions: ________________________________________________________________

________________________________________________________________

________________________________________________________________

Potential Routes of Exposure (check all that apply)

___ Inhalation	___ Skin Absorbtion 	___ Ingestion	___ Injection

Other: ___________________________________________________________

Personal Protective Equipment (check all that apply)

___ Gloves (specify type): ___________________________________________

___ Splash goggles		___ Safety glasses		___ Face shield

___ Lab coat			___ Apron			___ Rubber boots

___ Half-face respirator*				___ Full face respirator*

___ Powered air-purifying respirator*		___ Supplied air respirator

___ Self-contained breathing apparatus (SCBA)

Other PPE (please describe): ________________________________________________________________

________________________________________________________________

________________________________________________________________

*Specify cartridge type: ______________________________________________

Please notify UAF Environmental Health and Safety if respiratory protection is needed.  Medical examinations and fit tests are required for most respirator users.

Engineering/Ventilation Controls

___ Fumehood use only			___ Local exhaust ventilation required

___ Glove box use required		___ Glove bag use required

___ Biosafety Cabinet use only (identify biosafety cabinet class)

___ Class I		___ Class II		___ Class III

___ Ground fault interrupter required		___ Grounding required

___ Shielding required				___ Relief valve required

___ Vacuum breakers required			___ Back-flow preventers required

___ Spill; containment required			___ Leak detection required

___ Monitoring required				___ Alarm(s) required

Other (please describe): ___________________________________________

_______________________________________________________________

_______________________________________________________________

_______________________________________________________________

Special Handling Procedures and Storage Requirements

___ Refrigerated storage			___ Freezer storage

___ Store with desiccant			___ Store under nitrogen

___ Light-sensitive				___ Water-reactive

___ Ventilated storage required		___ Spill containment required

Describe safe handling and storage procedures: ________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

Spill Response Procedures

Leaks or spills must be promptly reported to the UAF Fire Department (911 or 7721).  The caller should try to provide a detailed description of the spill, including the name and approximate quantity of the chemical(s) involved.  Do not put yourself or others at risk to gather this information if it is unknown.  The Fire Department is responsible for notifying UAF Environmental Health and Safety or Hazardous Materials Response Team, as deemed necessary.

Waste Disposal

The UAF non-radioactive hazardous materials Transfer Request form is used to communicate the need for chemical waste removal.  Chemical wastes are managed by UAF Facilities Services, hazardous materials division (phone 5487).

For radioactive waste removal, contact the UAF Radiation Safety Officer (phone 7807).

Basic in-lab non-radioactive waste management procedure checklist:

___ The waste container is labeled with the UAF Non-Radioactive Hazardous Materials Transfer Request adhesive tag.

___ The waste container is marked to identify its contents, including the quantity or percentage of its constituents.

___ The waste container remains closed (capped) at all times, unless adding or removing waste.

___ The “satellite” accumulation start date is identified on the waste container.

___ The waste is stored in secondary containment.

___ The waste is segregated from all incompatible materials.

___ Incompatible materials are never stored in the same container.

___ Waste storage areas are inspected weekly by departmental personnel.

___ Containers holding hazardous waste are in food condition and not subject to failure due to their contents.

___ Materials containing >500ppm VOC’s in excess of 26 gallons per container are stored in DOT specification containers (contact EH&S).

___ Completed Hazmat Transfer Request forms are sent to UAF EH&S to schedule waste removal when container is full.

Decontamination

Equipment and glassware decontamination procedures are as follows:

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Other Information: ____________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

LABORATORY INSPECTION CHECKLIST

Appendix 2

Laboratory Safety Inspection Checklist

This checklist is provided as a guideline to conduct internal safety surveys.  This summary does not list every possible hazard, but may be useful in performing periodic safety inspections.  All items are not always applicable to all facilities. Please contact UAF EH&S for assistance in correcting deficiencies.

Department Name:_________________	Area Inspected:_______________

Inspected by: _____________________	Date of Inspection: ___/___/___

Administrative

__Yes __No __N/A   1) Are employees aware of how to access and interpret Material Safety Data Sheets (MSDS's)?

__Yes __No __N/A   2) Are safety training records maintained and available for review by employees, EH&S, outside agencies?

__Yes __No __N/A   3) Is the Department's Chemical Hygiene Plan readily available to all employees?

__Yes __No __N/A   4) Are safety inspection reports maintained and available for review by employees, EH&S, outside agencies?

__Yes __No __N/A   5) Is an annual (or continuous) listing of the chemical inventory maintained and  available to employees, EH&S, outside agencies?

General Safety Concerns

__Yes __No __N/A   6) Has an emergency information sign been posted outside the lab, indicating department personnel to contact in the event of an emergency?

__Yes __No __N/A   7) Are rooms, cabinets, designated areas containing such materials as regulated hazardous substances, radioactive materials, and biohazardous materials, posted with the appropriate warning signs?

__Yes __No __N/A   8) Are all exits and aisles to the outside free from any obstructions?

Seismic Safety

__Yes __No __N/A   9) Does shelving have lips or restraints in place to prevent spillage?

__Yes __No __N/A  10) Is overhead storage minimized and restrained?

__Yes __No __N/A  11) Are cabinets and storage shelves over 42 inches in height braced or otherwise fastened to the wall to prevent their falling in the event of an earthquake?

Personal Protective Equipment

__Yes __No __N/A  12) Is the appropriate personal protective equipment required for the lab available and utilized?

___ Safety Glasses	___ Goggles	___ Face Shields	___ Gloves

___ Lab Coats	___ Aprons		___ Foot wear

Laboratory Equipment

__Yes __No __N/A  13) Are the eyewash and emergency shower stations free from any obstructions and have been tested within the last 12 months?

__Yes __No __N/A  14) Is the maximum operable sash height information posted on the chemical fumehood(s)?

__Yes __No __N/A  15) Is the maximum operable sash height information followed by all fumehood users?

__Yes __No __N/A  16) Is an airflow indicator located on the fumehood?

__Yes __No __N/A  17) Has the fumehood been tested within the last year?

__Yes __No __N/A  18) Is storage with the fumehood minimized and operations conducted at least six inches inside the fumehood face?

__Yes __No __N/A  19) Are the biological safety cabinets appropriate for the biohazard class of material being examined?

__Yes __No __N/A  20) Are the biological safety cabinets certified annually?

__Yes __No __N/A  21) Is non-ionizing radiation equipment such as lasers, microwaves, and ultraviolet light sources properly posted and shielded?

__Yes __No __N/A  22) Are vacuum systems that are capable of imploding protected with cages or barriers; are smaller vacuum systems taped?

__Yes __No __N/A  23) Glass dewars are wrapped or shielded?

__Yes __No __N/A  24) Vacuum pump belt guard is in place?

__Yes __No __N/A  25) Two pronged appliances are not with a five foot radius or directly located above a sink or flammable materials?

__Yes __No __N/A  26) GFI's (ground fault interrupters) are located on outlets within six feet of sinks?

Refrigerators

__Yes __No __N/A  27) Are food and beverages kept out of work areas and out of laboratory refrigerators?

__Yes __No __N/A  28) Is the proper type of refrigerator used i.e., explosion-proof for flammable liquids?

__Yes __No __N/A  29) The laboratory refrigerators are properly marked, prohibiting the storage of food or drink?

__Yes __No __N/A  30) Walk-in refrigerators can be opened from the inside?

__Yes __No __N/A  31) The refrigerator/freezer is free of chemical spills or contamination; all containers are labeled, stoppered or tightly closed?

Compressed Gases

__Yes __No __N/A  32) Are all cylinders properly secured in an upright position?

__Yes __No __N/A  33) Are protective caps in place when the cylinder is not in use?

__Yes __No __N/A  34) Are incompatible cylinders stored separately?

__Yes __No __N/A  35) Are the cylinders legibly marked to identify contents?

__Yes __No __N/A  36) Are gages of oxygen regulators marked with the words, "Use No Oil"?

__Yes __No __N/A  37) Are the regulators, connections and supply lines in good condition?

__Yes __No __N/A  38) Are flash arresters on flammable gas supplies for atomic absorbtion instruments, in-house propane gas lines, hydrogen and oxy-acetylene torch lines?

Hazardous Materials

__Yes __No __N/A  39) Are chemical containers properly labeled with the chemical name and hazard of the material?

__Yes __No __N/A  40) Are chemicals color-coded to identify proper storage location?

__Yes __No __N/A  41) Are chemicals stored according to hazard classification and compatibility?

__Yes __No __N/A  42) Are all containers of potential peroxide- forming chemicals dated upon receipt and utilized or disposed within one year?

__Yes __No __N/A  43) Are flammable liquids stored in flammable liquid storage cabinets or in closed metal safety cans whenever possible?

__Yes __No __N/A  44) No storage of corrosive chemicals above eye level?

Hazardous Waste

__Yes __No __N/A  45) Is all lab personnel aware of how to have hazardous waste picked up?

__Yes __No __N/A  46) Is the disposal of hazardous substances down the drain or in the normal trash prevented?

__Yes __No __N/A  47) Are lab personnel instructed to not dispose of chemicals by evaporation in the fumehood?

__Yes __No __N/A  48) Are the proper containers obtained and used for storing hazardous waste?

__Yes __No __N/A  49) Are all containers properly marked to identify their contents and constituent concentrations and labeled with the UAF Non-Radioactive Transfer Request tag?

__Yes __No __N/A  50) Is the "satellite accumulation start date" identified on the waste container?

__Yes __No __N/A  51) Are all containers kept tightly closed except when adding or removing waste?

__Yes __No __N/A  52) Are wastes properly segregated with regard to their chemical compatibility?

__Yes __No __N/A  53) Are liquid waste containers kept in secondary containment tubs?

__Yes __No __N/A  54) Are waste storage areas inspected weekly by lab personnel?

__Yes __No __N/A  55) Are waste minimization practices used in the laboratory?

__Yes __No __N/A  56) Is infectious/biohazardous waste properly bagged and autoclaved or incinerated?

__Yes __No __N/A  57) Are all "sharps" collected in puncture and leak resistant containers prior to decontamination/disposal?

__Yes __No __N/A  58) Is broken glass collected in puncture resistant containers, marked with the words "Broken Glass" and sealed before disposal?

LABORATORY SAFETY TRAINING RECORD

Appendix 3

Print this form and keep it with your Chemical Hygiene Plan.

This is to certify that the individuals listed below were trained on the following elements of the Chemical Hygiene Plan.

- Standard Operating Procedures

- Prior Approval Circumstances

- Conducting a Hazard Assessment

- Methods of Control

- Proper Laboratory Hood Use

- Exposure Detection

- Chemical Storage

- Laboratory Waste Disposal

- Emergency Notification Procedures

- Spill Response Procedures

- Emergency Preparedness

Date:___________	Location:__________________Instructor:_________________

| NAME (please print) | Department/Title | Initials |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |

Add additional pages as necessary.

Lab Close-out Survey Checklist

Appendix 4

Old equipment, books, and other personal property have been removed or properly disposed.

All chemicals are clearly labeled, including hazard identification (i.e.,flammable, corrosive, poison, oxidizer, water/air reactive). Caps are secured.

Secondary containers (beakers, flasks, vials, etc.) have been emptied and the contents properly disposed.  Water and air reactives are immersed in appropriate solutions, dessicant, etc.

Fume hoods are clean and all chemicals removed, including the fume hood cabinet.

Peroxidizable compounds are labeled with the last date opened and arrangements have been made for disposal.

Waste is properly labeled and prepared for transport.  Arrangements for hazardous waste disposal should be made by contacting Facility Services, Hazardous Materials Division at x 5487.

Arrangements have been made with other faculty or departments to accept the remaining chemicals.

Space Planning and Management contacted prior to departure at x6340.

Appendix 5  In Case of Emergency

### YOU ARE IN ROOM # _______ IN BLDG# _______ AT PHONE#_______

| EMERGENCY EVENT | FIRST ACTION | THEN DO THIS |
| --- | --- | --- |
| FIRE OR EXPLOSION | Call 911 | PULL ALARM, CLOSE DOORS, EVACUATE |
| CHEMICAL SPILL THAT IS or MIGHT BE LIFE THREATENING | Call 911 | EVACUATE TO NEARBY AREA, REMOVE VICTIMS CLOTHING, DOUSE W/ WATER -15 MINUTES |
| CHEMICAL SPILL NOT LIFE THREATENING | Attend to anyone who may have been contaminated, call FS Haz Mat at x5487 | SECURE SPILL AREA, CLOSE DOORS, NOTIFY NEIGHBORING AREAS. (See Section V.B. of this Plan) |
| RADIOACTIVE SPILL | Call 911 and RSO | DETAIN THOSE CONTAMINATED, CONSULT RADIATION SAFETY MANUAL |
| CARDIAC ARREST | Call 911 | ENSURE ACCESS TO THE AREA, BEGIN CPR IF QUALIFIED |
| ODD ODOR Gas Odor | Call CHO, Codes & Safety 5413, or UFD 7721 | IDENTIFY SOURCE IF POSSIBLE |
| EARTHQUAKE If a gas leak or electrical hazard is detected | DO NOT CALL Call 7000 (FS) | INSIDE- GET BENEATH STRUCTURE (desk, doorway) / OUTDOORS GET INTO OPEN AREA AFTER TREMOR. |
| SECURITY PROBLEM - PERSON OR ANIMAL | Call 6200 or 911 24 hrs/UAF Police | GET COMPLETE DESCRIPTION |
| UTILITY FAILURE | Call x7000 (FS) If after hours call Dispatch at x7721 | GIVE EXACT LOCATION OF UTILITY FAILURE. TURN OFF EQUIPMENT, MAJOR APPLIANCES. DO NOT USE OPEN FLAMES FOR LIGHTING. |
| MEDICAL ASSIST | Call 911 | COMPLETE THE APPROPRIATE REPORTING FORM* |

Employee = Worker's Compensation "First Report of Injury"Student/Visitor = UAF "Accident/Injury Report"

EMERGENCY ASSEMBLY POINT (OUTSIDE): ______________________________

Please keep this form in a prominent position in your office.

Hazardous Materials pickup and disposal procedures

Appendix 6

Hazardous materials are defined as those materials that are: flammable, corrosive, air or water reactive or toxic (see Definitions of Hazardous Materials below).  Facility Services (FS), Hazardous Materials Division, is charged with the responsibility of removing all hazardous materials, used and unused, from UAF facilities.  Materials picked up by FS are recycled, used by someone else, or disposed of at an EPA approved facility.  The complete disposal procedure is found under UAF Policy 601 Hazardous Materials Management Procedure.

TO HAVE HAZARDOUS MATERIALS REMOVED FROM YOUR AREA, YOU MUST:

- Communicate the need for the removal of hazardous materials or other chemicals by utilizing the UAF Hazardous Materials Transfer Request Forms which are available upon request from the FS Hazmat Division.

- Fill out and affix required hazardous material label on containers and promptly place in approved hazardous material collection areas.

- At the time when the accumulation of a material begins, the date should be clearly marked and visible for inspection (mark “satellite accumulation start date” on transfer label).

- Send the white and yellow copies of the three part UAF Hazardous Material Transfer Request Form to the FS Hazmat Section.

- Retain the pink copy of request form for department records.

- Upon receipt of the UAF Hazardous Material Transfer Request Form, the FS Hazmat Section will arrange for pick-up and transfer of all the materials listed on the form to an approved hazardous material accumulation site. The hazardous materials are removed from collection areas by FS Hazmat Section personnel.

PROCEDURES FOR PROPERLY COLLECTING, STORING, HANDLING AND TRANSPORTING HAZARDOUS MATERIALS:

- Keep all hazardous materials in appropriate closed containers with airtight lids. Do not store hazardous materials in a fume hood. Keep all hazardous material containers closed at all times except when adding or removing the material.

- Do not mix incompatible chemicals (i.e. oxidizers with flammables . . .).

- Do not mix hazardous materials with non-hazardous materials. It greatly increases waste disposal costs.

- Accurately label all containers with chemical names and exact content as well as date of purchase.  It is imperative to avoid producing containers whose contents are unknown. Such materials are very expensive to dispose of. If containers of unknown chemicals are found, please contact Facility Services Hazardous Materials Division to make special arrangements.

PACKAGING REQUIREMENTS FOR ALL HAZARDOUS MATERIALS

- The outside of the containers must be clean and free of chemical contamination.

- Use appropriate containers. All glass containers must be securely packaged to prevent breakage during transport.

- All containers of liquids must have screw lids and must not leak when inverted. Corks, cotton plugs, tape, or parafilm are not acceptable lids for containers of hazardous materials.

- If possible, use the same container for disposal of used material that held the new material originally.

- Metal cans are not acceptable for accumulating hazardous solvents - except for waste oil.

- Loose solid materials must be placed in a sealed container or in a cardboard box lined with two polyethylene bags.

- Containers storing hazardous materials must be kept closed, except when adding or removing contents.

DEFINITIONS OF HAZARDOUS MATERIALS

Hazardous materials are those that "could cause injury or death; or damage or pollute land, air, or water." Hazardous wastes are defined as substances that are ignitable (flammable), corrosive, toxic, explosive, or reactive, i.e., react with air, water, or acids or bases. Specific definitions are found in the Code of Federal Regulations: 40CFR part 261. These are summarized below.

Ignitable: This category contains materials that are easily combustible or flammable. This includes liquids that have a flash point less than 60C (140F), and non-liquids that are capable, under standard temperature and pressure, of causing fire through friction, absorption of moisture, or spontaneous chemical change and when ignited burn so vigorously and persistently that they create a hazard, and any ignitable compressed gas described in 40 CFR 173.300. Examples are solvents and spent solvents (acetone, benzene, ethyl acetate, ethyl ether, methanol, methyl isobutyl ketone, xylene); ignitable paint waste (some paint removers, brush cleaners, and stripping agents; epoxy resins and adhesives (epoxies, rubber cements and marine glues); inks containing flammable solvents, and some degreasers. For additional information see 40 CFR 261.21.

Corrosive: This category includes acids and bases or mixtures having a pH less than or equal to 2 or greater than or equal to 12.5, and materials that burn the skin or dissolve metals. Examples are strong mineral acids (chromic, sulfuric, hydrochloric, or nitric) strong alkalis (potassium hydroxide), rust removers, and acid or alkaline cleaning fluids. This category also includes solids that when mixed with water form solutions that are strongly acidic or basic (ferric chloride, sodium hydroxide). For additional information see 40 CFR 261.22

Reactive: This category includes materials that are unstable or undergo rapid or violent chemical reaction when exposed to air, water or other material, generate toxic gases or vapors when mixed with water or when exposed to pH conditions between 2 and 12.5 (as in the case with cyanide or sulfide containing materials), forms potentially explosive

mixtures with water, are capable of detonation or explosive reaction when heated or subjected to shock. Examples are acetyl chloride, chromic acid, cyanides, hypochlorides, organic peroxides, perchlorates, permanganates, sulfides, some plating materials and bleaches. For additional information see 40 CFR 261.23

Toxic: This category includes heavy metal compounds such as: arsenic, barium, cadmium, chromium, lead, mercury, silver, selenium, etc. Pesticides such as, Aldrin, arsenic pentoxide, arsenic trioxide, cacodylic acid, chlordane, copper cyanides, DDT, Dieldrin, dimethylcarbamoyl chloride, Endrin, Lindane, pentachlorophenol, strychnine, etc.

Pathogenic, Carcinogenic, Infectious, and Etiologic agents: Includes any material that directly cause health problems such as, "a viable microorganism, or its toxin, which causes or may cause disease in humans or animals" (41 CFR 173.134). Infectious waste includes bloodborne pathogens (for example, hepatitis or AIDS virus). For a more detailed definition see the UAF Biosafety Manual and Bloodborne Pathogens Exposure Control Plan, available from the Department of EH&S (phone 5413).

Sharps: Sharps are defined as any non-contaminated sharp object that can penetrate the skin, including, but not limited to: broken capillary tubes and pipettes, blades from power tools, glass microscope slides and cover plates, and hypodermic and non-hypodermic needles.
There may be other hazardous substance that are not included here. It is your responsibility to determine if the materials you use are hazardous to human health or the environment. If you have any doubt, tag it for pickup and Division of Hazardous Materials will dispose of it appropriately.

You can find information about your material by looking at the Material Safety Data Sheet, available from the manufacturer. You should have Material Safety Data Sheets accessible for all substances you utilize.

SELECT CARCINOGENS

Appendix 7 *

OSHA’s definition of Select Carcinogens includes substances on the following lists:

OSHA Regulated Carcinogens including substance such as (29 CFR…)

“13 Carcinogens” 	1910.1003			Acrylonitrile 			1910.1045

Inorganic Arsenic 	1910.1018			Asbestos			1910.1101

Benzene 		1910.1028			1,3-Butadiene		1910.1051

Cadmium		1910.1027			Coke oven emissions 	1910.1029

1,2-Dibromo-3-chloropropane 1910.1044  	Ethylene oxide		1910.1047

Formaldehyde	1910.1048			Methylene chloride		1910.1052

Methylenedianiline	1910.1050			Vinyl chloride			1910.1017

WHO IARC Monographs Database

Group 1 – List of Human Carcinogens

Group 2A – List of Probable Human Carcinogens

Group 2B – List of Possible Human Carcinogens

NTP’s most recent Report on Carcinogens:

List of Substances Known to be Human Carcinogens

List of Substances Reasonably Anticipated to be Human Carcinogens

The following a PARTIAL list of known or potential carcinogens:

| CHEMICAL NAME | CAS NUMBER | CATEGORY* |
| --- | --- | --- |
| 2-Acetylaminofluorene | 53-96-3 | NTPAHC OSHA 13 |
| Acrylonitrile | 107-13-1 | IARC 2B;NTPAHC |
| Actinimycin D | 50-76-0 | IARC 3 |
| Adriamycin | 23214-928 | IARC 2A;NTPAHC |
| Aflatoxins | 1402-68-2 | IARC 1;NTPHC |
| 2-Aminoanthraquinone | 117-79-3 | NTPAHC |
| o-Aminoazotoluene | 97-56-3 | IARC 2Bs |
| 4-Aminodiphenyl | 92-67-1 | IARC 1;NTPHC, OSHA 13 |
| 2-Amino-5-(5-nitro-2-furyl)-1,3,4-thiadiazole | 712-68-5 | IARC 2Bs |
| Amitrole | 61-82-5 | IARC 2B;NTPAHC |
| O-Anisidine | 90-04-0 | IARC 2Bs;NTPAHC |
| O-Anisidine hydrochloride | 134-29-2 | NTPAHC |
| Aramite | 140-57-8 | IARC 2Bs;NTPAHC |
| Arsenic | 7440-38-2 | IARC 1;NTPHC |
| Arsenic compounds | 7440-38-2 | NTPHC |
| Arsenic pentoxide | 1303-28-2 | IARC 1 |
| Arsenic trioxide | 1327-53-3 | IARC 1;NTPHC |
| Arsenic, inorganic compounds | 7440-38-2 | IARC 1;NTPHC |
| Asbestos |  | IARC 1;NTPHC |
| Auramine | 492-80-8 | IARC 2B |
| Azaserine | 115-02-6 | IARC 2Bs |
| Azathioprine | 446-86-6 | IARC 1;NTPHC |
| Benzene | 71-43-2 | IARC 1;NTPHC |
| Benzidine | 92-87-5 | IARC 1;NTPHC, OSHA 13 |
| Benzo(a)pyrene | 50-32-8 | IARC 2A |
| Benzo(b)fluoranthene | 205-99-2 | IARC 3 |
| Benzotrichloride | 98-07-7 | NTPAHC |
| Benzyl violet | 1694-09-3 | IARC 2Bs |
| Beryllium and compounds | 7440-41-7 | IARC 1;NTPAHC |
| Bis-(chloromethyl) ether | 542-88-1 | IARC 1;NTPHC, OSHA 13 |
| Bis-chloroethyl nitrosourea | 154-93-8 | IARC 2A;NTPAHC |
| 1,4-Butanediol dimethane-sulphonate | 55-98-1 | IARC 1;NTPHC |
| B-butyrolactone | 3068-88-0 | IARC 2Bs |
| Cadmium and compounds | 7440-43-9 | NTPAHC |
| Carbon tetrachloride | 56-23-5 | IARC 2B;NTPAHC |
| Chlorambucil | 305-03-3 | IARC 1;NTPHC |
| Chloramphenicol | 56-75-7 | IARC 2B |
| Chlorinated toluenes |  | IARC 1 |
| 1-(2-Chloroethyl)-3-cyclo-hexyl-1-nitrosourea | 13010-47-4 | IARC 1;NTPAHC |
| 4-Chloro-o-phenylenediamine | 95-83-0 | IARC 2Bs;NTPAHC |
| Chromium hexavalent compounds | 1333-82-0 | IARC 1;NTPHC |
| Cisplatin | 15663-27-1 | IARC 2A |
| Citrus red no. 2 | 6358-53-8 | IARC 2Bs |
| Cobalt-chromium alloy | 11114-92-4 | IARC 2B;NTPHC |
| Creosote(s) | 8001-58-9 | IARC 2Bs |
| p-Cresidine | 120-71-8 | IARC 2Bs;NTPAHC |
| Cupferron | 135-20-6 | NTPAHC |
| Cycasin | 14901-08-7 | IARC 2Bs;NTPAHC |
| Cyclophosphamide | 50-18-0 | IARC 1;NTPHC |
| Dacarbazine | 4342-03-4 | IARC 2B;NTPAHC |
| Daunomycin | 20830-81-3 | IARC 2Bs |
| DDT | 50-29-3 | IARC 2B;NTPAHC |
| N,N'-Diacetylbenzidine | 613-35-4 | IARC 2Bs |
| 2,4-Diaminoanisole sulfate | 39156-41-7 | IARC 2Bs;NTPAHC |
| 4,4'-Diaminodiphenyl ether | 101-80-4 | IARC 2Bs |
| 2,4-Diaminotoluene | 95-80-7 | IARC 2Bs;NTPAHC |
| Dibenz(a,h)acridine | 226-36-8 | IARC 2Bs;NTPAHC |
| Dibenz[a,j]acridine | 224-42-0 | IARC 2Bs;NTPAHC |
| Dibenz[a,h]anthracene | 53-70-3 | IARC 2B;NTPAHC |
| Dibenzo(a,e)pyrene | 192-65-4 | IARC 2Bs |
| Dibenzo[a,h]pyrene | 189-64-0 | IARC 2Bs;NTPAHC |
| Dibenzo([a,i]pyrene | 189-55-9 | IARC 2Bs;NTPAHC |
| Dibenzo[q,i]pyrene | 189-55-9 | IARC 2Bs;NTPAHC |
| Dibromochloropropane (DBCP) | 96-12-8 | IARC 2Bs;NTPAHC |
| 3,3'-Dichlorobenzidine | 91-94-1 | IARC 2B;NTPAHC, OSHA 13 |
| Dichlorodiaminodiphenyl ether | 284-34-86-8 | IARC 2Bs |
| Diethyl sulfate | 64-67-5 | IARC 2A;NTPAHC |
| Diethylstilbestrol | 56-53-1 | IARC 1;NTPHC |
| Dihydrosafrole | 94-58-6 | IARC 2Bs |
| 3,3'-Dimethoxybenzidine | 119-90-4 | IARC 2B;NTPAHC |
| trans-2((Dimethylamino)methylimino)5- (2-(5-nitro-2-furyl)vinyl)-1,3,4-oxadiazole | 55738-54-0 | IARC 2Bs |
| 1,1-Dimethylhydrazine (UDMH) | 57-14-7 | IARC 2Bs;NTPAHC |
| Dimethyl sulfate | 77-78-1 | IARC 2A;NTPAHC |
| Dimethylaminoazobenzene | 60-11-7 | IARC 3;NTPAHC, OSHA 13 |
| Dimethylcarbamoyl chloride | 79-44-7 | IARC 2A;NTPAHC |
| Dioxane | 123-91-1 | IARC 2B;NTPAHC |
| Estrone | 53-16-7 | IARC 2B;NTPAHC |
| Ethyl methanesulfonate (EMS) | 62-50-0 | IARC 2B |
| Ethylene dibromide (EDB) | 106-93-4 | IARC 2B;NTPAHC |
| Ethylene thiourea | 964-45-7 | IARC 2B;NTPAHC |
| Ethyleneimine | 15-15-64 | OSHA 13 |
| Formaldehyde | 50-00-0 | IARC 2B;NTPAHC |
| 2-(2-Formylhydrazino)-4-(5-nitro-2-furyl)thiazole | 3570-75-0 | IARC 2B |
| Glycidylaldehyde | 765-34-4 | IARC 2B |
| Hexachlorobenzene | 118-74-1 | IARC 2B;NTPAHC |
| Hexamethyl phosphoramide | 680-31-9 | IARC 2B;NTPAHC |
| Hydrazine, sulfate (1:1) | 10034-93-2 | NTPAHC |
| Lasiocarpine | 303-34-4 | IARC 2B |
| Lead acetate | 301-04-2 | NTPAHC |
| Lead chromate(VI) oxide | 18454-12-1 | IARC 1;NTPHC |
| Lindane, and mixed isomers | 58-89-9 | NTPAHC |
| Melphalan | 148-82-3 | IARC 1;NTPHC |
| Merphalan | 531-76-0 | IARC 2B |
| Mestranol | 72-33-3 | IARC 2B;NTPAHC |
| Methyl chloromethyl ether | 107-30-2 | OSHA 13 |
| 4,4'-Methylene bis-(2-chloroaniline) | 101-14-4 | IARC 2B;NTPAHC |
| 4,4'-Methylene bis(N,N-dimethyl)benzeneamine | 101-61-1 | NTPAHC |
| 4,4'-Methylene bis (2-methylaniline | 838-88-0 | IARC 2A |
| Methylenedianiline | 101-77-9 | NTPAHC |
| Methylenedianiline dihydro-chloride | 13552-44-8 | NTPAHC |
| Methyl iodide | 74-88-4 | IARC 3;NTPAHC |
| Methyl methanesulfonate (MMS) | 66-27-3 | IARC 2A |
| Methylnitroanthraquinone | 129-15-7 | IARC 2B |
| Methylnitronitrosoguanidine | 70-25-7 | IARC 2A |
| Methylazoxymethanol acetate | 592-62-1 | IARC 2B |
| Methylthiouracil | 56-04-2 | IARC 2B |
| Metronidazole | 443-48-1 | IARC 2B;NTPAHC |
| Michler's ketone | 90-94-8 | NTPAHC |
| Mirex | 2385-85-5 | IARC 2B;NTPAHC |
| Mitomycin C | 50-07-7 | IARC 2B |
| Monocrotaline | 315-22-0 | IARC 2B |
| Mustard gas | 505-60-2 | IARC 1;NTPHC |
| alpha-Naphthylamine | 134-32-7 | OSHA 13 |
| beta-Naphthylamine (2-Naphthylamine) | 91-59-8 | IARC 1;NTPHC, OSHA 13 |
| 5-Nitroacenaphthene | 602-87-9 | IARC 2B |
| 4-Nitrobiphenyl | 92-93-3 | OSHA 13 |
| 5-Nitro-o-anisidine | 99-59-2 | NTPAHC |
| N-Nitrosodimethylamine | 62-75-9 | OSHA 13 |
| beta-Propiolactone | 57-57-8 | OSHA 13 |

IARC  International Agency for Research on Cancer

Group 1	The agent (mixture) is carcinogenic to humans.  The exposure circumstances entail exposures that are carcinogenic to humans.

Group 2A	The agent (mixture) is probably carcinogenic to humans.  The exposure circumstance entails exposures that are probably carcinogenic to humans.

Group 2B	The agent (mixture) is possibly carcinogenic to humans.  The exposure circumstance entails exposures that are possibly carcinogenic to humans.

Group 3	The agent (mixture or exposure circumstances) is unclassifiable as to carcinogenicity in humans.

Group 4	The agent (mixture, exposure circumstance) is probably not carcinogenic to humans,

NTP  National Toxicity Program

AHC	Anticipated Human Carcinogen

HC	Known Human Carcinogen

OSHA - Occupational Safety and Health Administration

13 OSHA-Regulated carcinogens

Laboratory emergency procedures during power outages

Appendix 8

It is important to remember that some equipment cannot be turned off and certain other pieces of equipment do not shut themselves off when there is a power outage.  Pre-plan specific procedures for your laboratory while adhering to the following:

- Close chemical fume hood sashes. No work is allowed in fume hoods during a power outage.

- Ensure that all chemical containers are secured with caps, parafilm, etc.

- All non-essential electrical devices should be turned off. Keep the doors of refrigerators and freezers closed. Check to ensure large lasers, radio frequency generators, etc have been turned off.

- Turn off all gas cylinders at the tank valves.

- If a low flow of an inert gas is being used to "blanket" a reactive compound or mixture, it may be appropriate to leave the flow of gas on.  The decision to do this should be part of the written Standard Operating Procedure specific for each lab in Appendix 1.

- Check all cryogenic vacuum traps (N2, CO2 and solvent). The evaporation of trapped materials may cause dangerous conditions.

- Check all pressure, temperature, air, or moisture sensitive materials and equipment. This includes vacuum work, distillations, glove boxes used for airless /moistureless reactions.

REQUIREMENTS FOR STORAGE AND HANDLING OF FLAMMABLE AND COMBUSTIBLE LIQUIDS - Appendix 9

| CLASS | IA | IB | IC | II |
| --- | --- | --- | --- | --- |
| Flash point | less than 73F | less than 73F | 73 - 100 F | 100 - 140F |
| Boiling point | less than 100F | greater than 100F |  |  |
| Flammability Potential | Extremely High | Very High | High | Moderate |
| EXAMPLES OF COMMONLY USED MATERIALS | acetaldehyde benzoyl peroxide ethyl ether pentane methyl formate | acetone ethanol butylamine gasoline methanol isopropanol | amyl acetate butanol chlorobenzene turpentine xylene | formaldehyde hydrazine kerosene |
| NFPA 704 HAZARD RATINGS * |  |  |  |  |
| MAXIMUM CONTAINER SIZE Glass | 1 pint (500 ml) | 1 quart ( 1 liter) | 1 gallon ( 4 liters) | 1 gallon (4 liters) |
| Metal or approved plastic | 1 gallon | 5 gallons | 5 gallons | 5 gallons |
| Safety cans | 2 gallons | 5 gallons | 5 gallons | 5 gallons |
| Metal drums (DOT) | N/A | 5 gallons | 5 gallons | 60 gallons |

NFPA is the acronym for the National Fire Protection Association. NFPA 704, Standard System for the Identification of the Fire Hazards of Materials, provides planning guidance to fire departments for safe tactical procedures in emergency operations, and gives on-the-spot information to safeguard the lives of fire fighting personnel and the others who may be exposed. The Hazard Identification System is not intended to identify the nonemergency health hazards of chemicals.

STORAGE REQUIREMENTS

- Flammable and/or combustible liquids stored in the open in a laboratory work area or inside any building shall be kept to the minimum necessary for the work being done.

- Maximum quantity permitted in labs and other areas of use is limited to a total of 10 gallons, all classifications combined, outside of a flammable storage cabinet or approved flammable storage room. Please refer to the table above.

- Quantities stored in flammable storage cabinets shall be limited to 60 gallons of class I or II liquids and the total of all liquids shall not exceed 120 gallons. Please refer to the table above for maximum allowable container size for each class. Not more than three cabinets shall be located in the same room.

- Quantities exceeding the above must be stored in an approved flammable storage room meeting the requirements of the Uniform Building and Fire Codes.

- Flammable and combustible liquids shall not be stored near exit doorways, stairways, in exit corridors, or in a location that would impede egress from the building.

- Flammable aerosols and unstable liquids shall be treated as class I-A liquids. Please refer to the table above.

- Materials which will react with water or other liquids to produce a hazard shall be segregated from flammable and/or combustible liquids.

HANDLING AND DISPENSING

1 Class I liquids shall not be transferred from one vessel to another in any exit passageway.

2 Transfer of flammable liquids from 5 gallon containers (or less) to smaller containers shall be done in a laboratory fume hood or in an approved flammable liquid storage room.

CHEMICAL INVENTORY Appendix 10

The UAF Chemical Inventory Worksheet is a separate attachment that should be completed according to the instructions on the worksheet and added to this CHP.  It is an Excel Document printed on 8.5” x 14” paper that can be found on EH&S web page, or you may call Leah Boback, UAF Industrial Hygienist, at x5812 for a copy.